INFORMATION SHEET

Candles and other Open-Flame Decorative Devices in Places of Assembly

A permit shall be obtained from the Fire Prevention Bureau to use open flames or candles in connection with assembly areas, dining areas of restaurants or drinking establishments.     UFC 105.8.c.1

- Candles and other open-flame devices shall not be used in places of assembly or in drinking or dining establishments.   UFC  2501.17

- EXCEPTIONS:

- ·	When used in conjunction with approved heating or cooking appliances in areas not accessible to the public.

- ·	When used in conformance with Section 1109.8.

- ·	When used in conformance with Section 8203.2.1.8

- Candles and other open-flame decorative devices shall be in accordance with the following: UFC 1109.8.2

- ·	Classes-I and -II liquids and LP-gas shall not be used.

- ·	Liquid- or solid-fueled lighting devices containing more than 8 ounces must self-extinguish and not leak fuel at a rate of more than 1/4 teaspoon per minute if tipped over.

- ·	The devices or holder shall be constructed to prevent the spillage of liquid fuel or wax at the rate of more than 1/4 teaspoon per minute when the device or holder is not in an upright position.

- ·	The device or holder shall be designed so that it will return to the upright position after being tilted to an angle of 45 degrees from vertical.

- EXCEPTION:	Units that self-extinguish if tipped over and do not spill fuel or wax at the rate of more than 1/4 teaspoon per minute if tipped over.

- ·	The flame shall be enclosed except as follows:

- ·	Openings on the side shall not be more than 3/8 inch diameter.

- ·	Openings on the top and the distance to the top shall be such that a piece of tissue paper placed on the top will not ignite in 10 seconds.

- ·	Chimneys shall be made of noncombustible materials. Such chimneys shall be securely  attached to the open-flame device.

- EXCEPTION:	The chimney need not be attached to any open-flame device that will self-extinguish if the device is tipped over.

- ·	Fuel canisters shall be safely sealed for storage.

- ·	Storage and handling of combustible liquids shall be in accordance with Axticle 79.

- ·	Shades, if used, shall be made of noncombustible materials and securely attached to the open-flame device holder or chimney.

- ·	Candelabras with flame-lighted candles shall be securely fastened in place to prevent overturning and located away from occupants using the area and away from possible contact with drapes, curtains or other combustibles.

- When, in the opinion of the chief, adequate safeguards have been taken, participants in religious ceremonies are allowed to carry hand-held candles.  Hand-held candles shall not be passed from one person to another while lighted. UFC 11 09.8.3

- When approved, open-flame devices used in conjunction with theatrical performances are allowed to be used when adequate safety precautions have been taken.

- UFC 11 09.8.4

- Flammable or combustible liquids used in the preparation of flaming foods or beverages shall be dispensed from one of the following: UFC 1109.9.2

- A I -ounce container or

- A container not exceeding 1 quart capacity with a controlled-pouring device that will limit the flow to a 1-ounce serving.

- Containers shall be secured to prevent spillage when not in use.  UFC 1109.9.2.1

- The serving of flaming foods or beverages shall be done in a safe manner and shall not create high flames.  The pouring, ladling or spooning of liquids is restricted to a maximum height of 8 inches above the receiving receptacle.  UFC 11 09.9.2.2

- Flaming foods or beverages shall be prepared only in the immediate vicinity of the table being serviced.  They shall not be transported or carried while burning.   UFC 11 09.9.3

- The person preparing the flaming foods or beverages shall have a wet cloth-towel immediately available for use in smothering the flames in the event of an emergency.  UFC 1109.9.4

The items listed are basic information only.  Codes are subject to change.  Additional requirements may apply.